Department Of Second Year Engineering Academic Year 2025-26

SEM-III

# Case Study Report

Subject : Digital Electronics and Logic Design

Subject Code : MDM-230-COM

Unit II: Combinational Logic Design

Title:	Design	of	a	Binary	Calculator	for	BCD	Input	and	2’s Complement Operations

Roll No. : 72

Division : C

Submitted by :	Subject Teacher :

Divya Walke	Prof. Shrishail Patil

Index

| Sr. No. | Topics |
| --- | --- |
| 1 | Introduction |
| 2 | Overview of Number Systems in Digital Design |
| 3 | Core Combinational Logic Design: BCD Adder/Subtractor |
| 4 | 2’s Complement Subtraction Implementation |
| 5 | System Block Diagram and Functionality |
| 6 | Testing and Verification (Sequential Flow) |
| 7 | Conclusion |

# Introduction

Digital Electronics and Logic Design (DELD) principles are fundamental to building Arithmetic Logic Units (ALUs) found in every modern processor. This case study details the design of a basic digital calculator capable of performing both addition and subtraction on numbers. To ensure human readability and efficient arithmetic, the design incorporates two critical digital concepts:

- Binary-Coded Decimal (BCD): Used for input/output to interface easily with decimal displays (like 7-segment displays).

- 2’s Complement Arithmetic: Used for efficient subtraction by converting it into an addition problem, allowing a single circuit to handle both operations.

The goal is to design the core combinational logic that can accept two BCD digits and a control signal, producing the correct BCD result and handling signed operations via the 2’s complement method.

# Overview of Number Systems in Digital Design Binary-Coded Decimal (BCD)

BCD is a common method for encoding decimal numbers in binary form. Each decimal digit (0 through 9) is represented by its 4-bit binary equivalent.

  - Advantage: Simpler conversion for decimal input/output devices (like digital displays).

  - Design Requirement: Logic must include a BCD correction circuit, as a standard 4-bit binary adder will produce an incorrect binary sum if the result is greater than 9.

# 2’s Complement Representation

The 2’s complement system is used to represent signed (positive and negative) binary numbers.

  - Rule: Subtraction is implemented as addition of the minuend and the 2’s complement of the subtrahend:

𝐴 − 𝐵 = 𝐴 + (2’s Complement of 𝐵)

  - Implementation: The 2’s complement of a number 𝐵is found by inverting all its bits (1’s complement) and adding 1. This requires a dedicated logic circuit (inverter and a full adder at the LSB).

# Core Combinational Logic Design: BCD Adder/Subtractor

The core of the calculator is a circuit that can perform BCD addition.

# 4-bit Binary Adder

The design starts with a 4-bit Parallel Binary Adder (four cascaded Full-Adder circuits). This handles the binary sum of two 4-bit numbers 𝐴and 𝐵, producing a 4-bit sum 𝑆and a carry-out

𝐶out.

# BCD Correction Logic

If the sum output 𝑆from the binary adder is greater than 9 (i.e., 1010to 1111), or if 𝐶outis 1, a decimal carry is generated and the sum is incorrect in BCD.

  - Correction: Add 01102(decimal 6) to the binary sum 𝑆.

  - Implementation: Requires additional logic to detect invalid BCD sums and a second 4-bit binary adder to perform the +6 correction.

# BCD Adder Logic:

If (𝑆 > 9 OR 𝐶out = 1): 𝑆BCD = 𝑆 + 01102

Else: 𝑆BCD = 𝑆

# 2’s Complement Subtraction Implementation

To enable subtraction, the circuit is modified into a selectable Adder/Subtractor using a control signal 𝑀.

| Control Signal (M) | Operation |
| --- | --- |
| 0 | Addition (A + B) |
| 1 | Subtraction (A - B) |

When 𝑀 = 1(Subtraction Mode):

- 1’s Complement: The 𝐵input bits (𝐵0 to 𝐵3) are passed through XOR gates with 𝑀 = 1as the second input. Since 1 ⊕ 𝐵 = 𝐵¯ , this generates the 1’s complement (𝐵¯ ).

- Adding 1 for 2’s Complement: The control signal 𝑀 = 1is directly connected to the Carry-In (𝐶in) of the LSB Full Adder. Adding 𝐴 + 𝐵¯ + 1successfully computes 𝐴 + (2’s Complement of 𝐵) = 𝐴 − 𝐵.

| Logic Component | Input B | Control M | Output 𝐵mod |
| --- | --- | --- | --- |
| XOR Gate Array | 𝐵i | 𝑀 | 𝐵i ⊕ 𝑀 |
| LSB Full Adder | 𝐶in | N/A | 𝑀 |

# System Block Diagram and Functionality

The final system integrates the BCD logic and the 2’s complement logic.

  - Inputs: Two 4-bit BCD numbers (A and B) and a 1-bit control signal (M).

  - Outputs: A 4-bit BCD Sum/Difference (𝑆BCD) and a Final Carry/Borrow (𝐶Final).

| Component | Function |
| --- | --- |
| M-Control Logic (XORs) | Generates 𝐵or 𝐵¯ based on M |
| 4-bit Binary Adder 1 | Performs 𝐴 + 𝐵mod + 𝑀(initial sum) |
| Correction Logic | Detects 𝑆 > 9or 𝐶out = 1 |
| 4-bit Binary Adder 2 | Performs the +6 BCD correction if needed |

# Testing and Verification (Sequential Flow)

To verify the design, consider two examples — one addition and one subtraction.

| Scenario | M | Input A | Input B | Binary Operation (A B) |  |  | ± | 𝐶out | BCD Correction | 𝑆BCD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition | 0 | 5 | 7 | 0101 | + | 0111 | = | 0 | 1100 > 9 → | 12 (0010, |
|  |  | (0101) | (0111) | 1100 |  |  |  |  | +0110 | C=1) |
| Subtraction | 1 | 9 | 4 | 9 + 2’s Comp(4) |  |  |  | 1 | 1001 + 1100 = | 5 (0101, |
|  |  | (1001) | (0100) |  |  |  |  |  | 0101 | C=1) |

# Subtraction Details (9 – 4):

- B Input: 𝐵 = 0100. With 𝑀 = 1, 𝐵mod = 1011(1’s complement).

- LSB 𝐶in: 𝑀 = 1.

- Addition: 𝐴 + 𝐵mod + 𝐶in = 1001 + 1011 + 1 = 10101.

- Result: The sum is 0101with a final carry (𝐶out = 1). The 𝐶out = 1indicates a positive result, and the result 0101is 510, which is correct.

# Conclusion

This case study successfully designed a versatile digital calculator core by integrating fundamental DELD concepts. The use of a 4-bit BCD Adder with correction logic ensures the output is compatible with decimal display systems, while the implementation of the 2’s complement technique allows a single, optimized circuit (using only XOR gates and a single carry-in modification) to perform both addition and subtraction. Such combined functionality is the basis for the arithmetic circuits within microprocessors and microcontrollers, highlighting the direct relevance of these logic design principles to complex digital systems.